COMSATS University Islamabad, Lahore Campus

Department of Electrical and Computer Engineering

# CPE231 –Electronic Devices and Circuits

# Lab Manual for Spring 2024

Lab Resource Person

Ms. Wajeeha Khan

Theory Resource Person

Ms. Wajeeha Khan

Supervised By

Dr. Jehangir Arshad

Name: __________________________ Registration Number: CUI/ - - /LHR

Program: _______________________ Batch: _____________________________________

Semester ___________________________

Revision History

| S. No. | Activity | Date (MM/YYYY) | Performed by |
| --- | --- | --- | --- |
| 1 | Lab Manual Preparation | 02/2019 | Engr. Muzamil Ahmad Zain |
| 2 | Lab Manual review | 03/2019 | Dr. Khurram Shabih Zaidi |
| 3 | Lab Manual Formatting | 02/2020 | Syed Junaid Akhtar /Muzamil Ahmad |

Preface

This manual is intended for use in semiconductor devices course namely Electronic Devices and Circuits. The manual contains sufficient exercises for a typical 14 week course using a three hour practicum period. The topics cover three major portions namely Diodes and its applications, Bipolar Junction Transistors and Field Effect Transistors. The course starts with the Diode applications, the next section covers the small signal analysis of the BJT & FET based amplifiers. Next section deals with low frequency response of BJT, the multistage amplifier and Operational Amplifier (and its various applications). For equipment, each lab station includes a dual adjustable DC power supply, a dual trace oscilloscope, a function generator and a quality DMM. For components, diodes (Si or Ge), transistors NPN (2N3904, 2N2219, or equivalent general purpose transistor), Op-Amp (741 or Equivalent), a selection of standard value ¼ watt carbon film resistors ranging from a few ohms to a few mega-ohms is required along with an array of typical capacitor values. The students are expected to design and test their circuit on breadboard using these components as per the requirements of each experiment.

Each exercise begins with an Objective and a Theory Overview. The Equipment List follows with space provided for serial numbers and measured values of components. Schematics are presented next along with the step-by-step procedure. Many exercises include sections on troubleshooting and design. All data tables are grouped together, typically with columns for the theoretical and experimental results, along with a column for the percent deviations between them. Finally, a group of appropriate questions are presented.

Books

Textbook

- Electronic Devices and Circuit Theory by Boylestad & Nashelsky (11th Edition) PEARSON.

Reference Books

- Electronic Devices-Conventional Current Version By Floyd (9th Edition)

- Schaum's Outlines Electronic Devices and Circuits

- Microelectronic Circuits by Sedra and Smith, 5th Edition, Oxford University Press

Learning Outcomes

Theory CLOs

After successfully completing this course, the students will be able to:

- Apply the basic concepts of semiconductor theory, flow of electrons and holes, current, voltage and passive circuit elements and manipulation of basic voltage, current and electrons/holes transfer in various semiconductor devices and electronic circuits (PLO1-C2)

- Analyze the diode-based circuits, transistor circuits, op-amp circuits and oscillator circuits for voltage gain, current gain and frequency response using principles of electrons/holes transfer theory in semiconductors applied to electronic devices and circuits (PLO2-C4)

- Design Zener diode based voltage regulator, op-amp based inverting and non-inverting amplifiers by applying the working principles of electronic devices (PLO3-C5)

Lab CLOs

After successfully completing this course, the students will be able to:

- Design simple dc and ac circuits containing diodes, BJTs and FETs using standard circuit analysis techniques. (PLO3-C5)

- Construct dc and ac circuits containing diodes, BJTs and FETs and measure their input and output voltages and currents using breadboard, digital multimeter (DMM), digital storage oscilloscope (DSO) and simulation tools. (PLO5-P3)

CLOs – PLOs Mapping

| PLO CLO | PLO1 | PLO2 | PLO3 | PLO5 | PLO10 | Cognitive Domain | Affective Domain | Psychomotor Domain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CLO1 | x |  |  |  |  | C2 |  |  |
| CLO2 |  | x |  |  |  | C4 |  |  |
| CLO3 |  |  | x |  |  | C5 |  |  |
| CLO4 |  |  | X |  |  | C5 |  |  |
| CLO5 |  |  |  | x |  |  |  | P4 |

Lab CLOs – Lab Experiment Mapping

| Lab CLO | Lab 1 | Lab 2 | Lab 3 | Lab 4 | Lab 5 | Lab 6 | Lab 7 | Lab 8 | Lab 9 | Lab 10 | Lab 11 | Lab 12 | Lab 13 | Lab 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CLO4 | x | x | x | x | x | x | x | x | x | x | x | x | x | x |
| CLO5 | x | x | x | x | x | x | x | x | x | x | x | x | x | x |

Grading Policy

The final marks for lab would comprise of Lab Assessment (25%), Lab Midterm  (25%), and Lab Terminal (50%).

| Lab Assessments | 25% |
| --- | --- |
| Mid Term: 0.5*(Midterm Exam result) + 0.5*[ (average of lab evaluation of Lab 1-7) * 2.5] | 25% |
| Terminal 0.5*(Terminal Exam result) +0.25*[(average of lab evaluation of Lab 9-12) *5] + 0.10*[(average of lab evaluation of Lab 5-8) *5] + 0.15*[(average of lab evaluation of Lab 1-4) *5] | 50% |
| Total (lab) | 100% |

The minimum pass marks for both lab and theory shall be 50%. Students obtaining less than 50% marks (in either theory or lab, or both) shall be deemed to have failed in the course. The final marks would be computed with 75% weight to theory and 25% to lab final marks.

List of Equipment

| S.No | Name of Equipment |
| --- | --- |
| 1 | Analog / Digital Trainers A-Tek (Model AT-700)(ULT-3000) |
| 2 | Function Generators LoadStar (Model FG-2100A) |
| 3 | DC Power Supplies (Model DF1730SL3A) 0-30Vdc 3A |
| 4 | Multimeter Digital Model M-3900 & UT55 |
| 5 | DIGITAL OSCILLOSCOPES INSTEK (GDS-1052-U) |
| 6 | Soldering & Desoldering Station |

Components

- Resistors

- Capacitors

- Transistors

- NPN (2N3904, 2N2219, or equivalent general purpose transistor)

- TIP 120 (npn Darlington)

- 2N3823 (JFET n-channel transistor 2N3823, or equivalent

- NPN Medium Power, 15W (2N4300 or equivalent) PNP medium Power, 15W (2N5333 or equivalent) Silicon Diode

- Zenor diode

- Op-Amp (741 or Equivalent)

Software Resources

- PSpice

- LTspice

- Proteus

Laboratory Guidelines (Laboratory procedures)

Every week before lab, each student should read over the laboratory experiment and work out the various calculations, etc. that are outlined in the pre-lab.

- Return parts and jumper wires to correct bins when you are finished with them.

- Do not put suspected defective parts back in the bins. Give them to the Lab Technician for testing or disposal.

- Report all equipment problems to Lab Instructor or Lab Technician.

- Most experiments have several parts; students must alternate in doing these parts as they are expected to work in group.

- Each student must have a laboratory notebook. The notebook should be a permanent document that is maintained and witnessed properly, and that contains accurate records of all lab sessions.

- Laboratory and equipment maintenance is the responsibility of not only the Lab Technician, but also the students. A concerted effort to keep the equipment in excellent condition and the working environment well-organized will result in a productive and safe laboratory.

Safety in the Laboratory

To minimize electric shock hazard, the experiments are designed for low-voltage; however one should never assume that electric circuits are safe. Few milliamps of current through the body can be lethal. For your safety you must follow safety rules particularly:

- Turn off power before working on circuits.

- Know the location of emergency power-off switch.

- Make sure that the transformers and equipments are plugged into utility lines, have no exposed wiring. Check with the instructor if you are not certain about the procedure.

- Take care when using power supplies, which may be low voltage but can supply currents in the ampere range. Shorting such a supply can lead to a serious burn as high currents arc and can ignite flammable material. This is precisely why a car battery needs to be treated with respect. The hundreds of amps a battery can supply are sufficient to cause serious burns.

- The equipment is heavy enough to be generally stable on the bench. Be sure to keep the equipment away from the edges of the benches to avoid having a piece of equipment fall off the bench. Besides endangering people who might be struck, falling equipment endangers everyone in vicinity by stressing the power cords, possibly causing a line short or live fault on the equipment, not to mention damage to the expensive lab equipment. In general electronic equipment does not survive harsh treatment.

Laboratory Notebook

The laboratory notebook is a record of all work pertaining to the experiment. This record should be sufficiently complete so that you or anyone else of similar technical background can duplicate the experiment and data by simply following your laboratory notebook. Record everything directly into the notebook during the experiment. Do not use scratch paper for recording data. Do not trust your memory to fill in the details at a later time.

GUIDELINES FOR LABORATORY NOTEBOOK

- State the objective of the experiment.

- Draw the circuit diagram and mention the values of resistances etc. which are used.

- Make a note of all the measuring instruments you have used.

- Mention the formulas used.

- Create a table and write down the readings, including the units.

- Show all your calculation neatly and SYSTEMATICALLY. Do this is an organized manner.

- Attach graph if any.

- Be concise. Complete sentences are not necessary as long as the context is clear.

- If mistakes are made, they should not be erased. Just bracket them and make a short note explaining the problem.

- Make entries as the lab progresses; don't assume you can fill it in later. The instructor will ask to see it during the lab.

- Date every page.

- All important results must be underlined.

- Attach simulation and hand calculation to your note book.

- Draw the figure using pencil before you come to the lab so that you can make corrections to it in case you need to do so by erasing and redrawing. This will ensure tidy and neat work.

- Prepare the READING TABLE using pencil and ruler and not just by sketching lines. Sketching gives rise to crooked lines and gives the lab notebook a haphazard look.

- Take a few short notes (2-3 lines), which explains some of the problems you encountered while doing the experiment. This will help you write better reports.

General Lab Report Format

Following the completion of each laboratory exercise in Engineering courses, a report must be written and submitted for grading. The purpose of the report is to completely document the activities of the design and demonstration in the laboratory. Reports should be complete in the sense that all information required to reproduce the experiment is contained within. Writing useful reports is a very essential part of becoming an engineer. In both academic and industrial environments, reports are the primary means of communication between engineers.

There is no one best format for all technical reports but there are a few simple rules concerning technical presentations which should be followed. Adapted to this laboratory they may be summarized in the following recommended report format:

- Title page

- Introduction

- Experimental Procedure

- Experimental Data

- Discussion

- Conclusions

Detailed descriptions of these items are given below.

• Title Page:

The title page should contain the following information

  - Your name

  - ID

  - Course number (including section)

  - Experiment number and title

  - Date submitted

  - Instructors Name

- Introduction:

It should contain a brief statement in which you state the objectives, or goals of the experiment. It should also help guide the reader through the report by stating, for example, that experiments were done with three different circuits or consisted of two parts etc. or that additional calculations or data sheets can be found in the appendix, or at the end of the report.

• The Procedure:

It describes the experimental setup and how the measurements were made. Include here circuit schematics with the values of components. Mention instruments used and describe any special measurement procedure that was used.

• Results/Questions:

This section of the report should be used to answer any questions presented in the lab handout. Any tables and/or circuit diagrams representing results of the experiment should be referred to and discussed/explained with detail. All questions should be answered very clearly in paragraph form. Any unanswered questions from the lab handout will result in loss of points on the report.

The best form of presentation of some of the data is graphical. In engineering presentations a figure is often worth more than a thousand words. There are some simple rules concerning graphs and figures which should always be followed. If there is more than one figure in the report, the figures should be numbered. Each figure must have a caption following the number. For example, “Figure 1.1: TTL Inverter” In addition, it will greatly help you to learn how to use headers and figures in MS Word.

• The Discussion:

It is a critical part of the report which testifies to the student’s understanding of the experiments and its purpose. In this part of the report you should compare the expected outcome of the experiment, such as derived from theory or computer simulation, with the measured value. Before you can make such comparison you may have to do some data analysis or manipulation.

When comparing experimental data with numbers obtained from theory or simulation, make very clear which is which. It does not necessarily mean that your experiment was a failure. The results will be accepted, provided that you can account for the discrepancy. Your ability to read the scales may be one limitation. The value of some circuit components may not be well known and a nominal value given by the manufacturer does not always correspond to reality. Very often, however, the reason for the difference between the expected and measured values lies in the experimental procedure or in not taking into account all factors that enter into analysis.

• Conclusion:

A brief conclusion summarizing the work done, theory applied, and the results of the completed work should be included here. Data and analyses are not appropriate for the conclusion.

Notes

Typed Reports are required. Any drawings done by hand must be done with neatness, using a straight edge and drawing guides wherever possible. Free hand drawings will not be accepted.

Pre-lab results should be reported in the provided sheets at the end of the manual. It is your responsibility to obtain the instructor’s signature and to include the signed sheet with your final experiment report. Each student must submit an individual report based on an individual effort.

# Table of Contents

Table of Contents	11

LAB # 1: To understand how to generate a signal using function generator and display using oscilloscope and construct a diode based circuit and display the output using hardware tools	14

Objectives	14

Lab Task-1:	14

Pre-Lab	14

In-Lab	16

Lab Task 1.1:	16

Lab Task 1.2:	16

Lab Task 1.3:	17

Lab Task 1.4:	18

Lab Task-2:	20

Pre-Lab: Familiarize yourself with diode	20

In-Lab	22

Lab Task 2.1:	22

Lab Task 2.2:	23

LAB #2: To construct a half wave and full wave rectifier circuit and display waveform using hardware tools	26

Objectives	26

Lab Task-1:	26

Pre-Lab	26

In-Lab	28

Lab Task 1.1:	28

Lab task 1.2:	29

Lab Task 1.3:	31

Lab Task 2:	34

Pre-Lab	34

In-Lab	35

Lab Task 2.1:	35

Lab Task 2.2:	37

Lab Task 2.3:	37

Lab Task 2.4:	39

LAB # 3: To construct a zener diode based circuit and sketch its I-V characteristics	41

Objectives	41

Pre-Lab	41

In-Lab	43

Lab Task 1:	43

Lab Task 2:	45

LAB # 4: To display the output of diode based clipper circuit using hardware tools	49

Objectives	49

Lab Task-1:	49

Pre-Lab	49

In-Lab	50

Task 1.1:	50

Task 1.2:	51

Task 1.3:	55

LAB # 5: To display the output of diode based clamper circuit using hardware tools	59

Objectives	59

Pre-Lab	59

In-Lab	60

Lab Task 1.1:	60

Lab Task 1.2:	65

LAB # 6: To Sketch the input and output characteristics of common base BJT Transistor using hardware tools	71

Objectives	71

Pre-Lab	71

In-Lab	73

Lab Task 1:	73

Lab Task 2:	76

LAB # 7: To sketch the input and output characteristics of Common Emitter BJT Transistor using hardware tools	79

Objectives	79

Pre-Lab	79

In-Lab	80

Lab Task 1:	80

Lab Task 2:	82

LAB # 8: To measure the quiescent operating point of Fixed and Emitter Biased BJTs using hardware tools	85

Objectives	85

Pre-Lab	85

In-Lab	86

Lab Task 1.1:	86

Lab Task 1.2:	86

Lab Task 2.1:	89

Lab Task 2.2:	90

LAB # 9:  To construct a voltage-divider biased common-emitter circuit and measure its Q point voltage and current using digital multimeter	92

Objectives	92

Pre-Lab	92

In-Lab	92

LAB # 10: To Design a Voltage-Divider Biased Common-Emitter Amplifier with Fully Bypassed Emitter Resistance and Analyze the effect of Load on its AC Operation Using Hardware and Software Tools	96

Objectives	96

Pre-Lab	96

In-Lab Tasks	98

LAB # 11: To Construct the Class A Power Amplifier and Compute its Efficiency Using Hardware and Software tools	103

Objectives	103

Pre-Lab	103

In-Lab Tasks	105

LAB # 12: To Design and Construct the Basic Linear Op-Amp Circuit using Hardware and Software Tools	110

Objectives	110

Pre-Lab	110

In-Lab Tasks	114

# LAB # 1: To understand how to generate a signal using function generator and display using oscilloscope and construct a diode based circuit and display the output using hardware tools

## Objectives

- Using oscilloscope to measure the amplitude and time period of voltage signal

- Use the function generator to generate and measure the amplitude and duration of a voltage signal

- To understand the behavior of a diode by constructing a circuit in forward and reverse biased configuration

- To measure and display the response of a diode based circuit

## Lab Task-1:

## Pre-Lab

Introduction to Oscilloscope

The oscilloscope is the most important instrument available to the practicing technician or engineer. It permits the visual display of a voltage signal that can reveal a range of information regarding the operating characteristics of a circuit or system that is not available with a standard multi-meter. At first glance the instrument may appear complex and difficult to master. Be assured, however, that once the function of each section of the oscilloscope is explained and understood and the system is used throughout a set of experiments, your expertise with this important tool will develop quite rapidly. In addition to the display of a signal, it can also be used to measure the average value, rms value, frequency, and period of a sinusoidal or non-sinusoidal signal. The screen is divided into centimeter divisions in the vertical and horizontal directions. The vertical sensitivity is provided (or set) in volts/div, while the horizontal scale is provided (or set) in time (s/div.). If a particular signal occupies 6 vertical divisions and the vertical sensitivity is 5mV/div. The magnitude of the signal can be determined from the following equation:

Amplitude of signal voltage = voltage sensitivity (V/div.) x deflection (div.)

V	VS = (5mV/div)(6 div) = 30m

If one cycle of the same signal occupies 8 divisions on the horizontal scale with horizontal sensitivity of 5µs/div., the period and frequency of the signal can be determined using the following equations:

Period of signal voltage = horizontal sensitivity(s/div) x deflection (div)

T = (5µs/div)(8 div) = 40µs

f = 1/T = 1/40µs = 25 kHz

Introduction to Function Generator

A function generator is usually a piece of electronic test equipment  used to generate different types of electrical waveforms over a wide range of frequencies. Some of the most common waveforms produced by the function generator are the sine, square, triangular and saw-tooth shapes. These waveforms can be either repetitive or single-shot. Integrated circuits used to generate waveforms may also be described as function generator ICs.

Function generators are used in the development, test and repair of electronic equipment. For example, they may be used as a signal source to test amplifiers or to introduce an error signal into a control loop.

Task

Read the Oscilloscope user manual and describe the function and use of each of the following controls or sections of the oscilloscope in your own words.

- Vertical and horizontal position controls

- Vertical Sensitivity:

- Horizontal sensitivity:

- Vertical mode selection:

- AC-GND-DC switch:

- Calibrate switches:

- Trigger section:

- External trigger input:

- Probe:

Describe one mistake that can damage a multimeter for each of the following measurements:

A. voltage

B. resistance

C. current

State whether propagation of errors can be used to find the uncertainty of:

A. a quantity measured directly using a multimeter

B. a calculated quantity based on more than one measurement

C. both of the above

## In-Lab

### Lab Task 1.1:

Procedure

- Turn on the oscilloscope and adjust the necessary controls to establish a clear, bright, horizontal line across the center of the screen. Do not be afraid, to adjust the various controls to see their
effects on the display.

- Connect the function generator to, one vertical channel of the oscilloscope and set the output of the generator to a 1000 Hz sinusoidal waveform.	.

- Set the vertical sensitivity of the scope to 1 V/div. and adjust the amplitude control of the function generator to establish a 4 V peak to-peak (p-p) sinusoidal waveform on the screen.

### Lab Task 1.2:

Horizontal Sensitivity

- Determine the period of the 1000 Hz sinusoidal waveform in milliseconds using the equation T = 1/f. Show all work for each part of the experiment. Be neat.

T (calculated) =____________

- Set the horizontal sensitivity of trio scope to 0.25 ms/div. Using the results of Part 2(a) predict and calculate the number of horizontal divisions required to properly display one full cycle of the 1000 Hz signal.

Number of divisions (calculated) = ___________

- Use the oscilloscope measure the number of required divisions and insert below. How does the     result compare to the calculated number of divisions.

Number of divisions (measured)   = ___________

- Change the horizontal sensitivity of the oscilloscope to 0.5 ms/div without touching any of the controls of the function generator. Using the results of Part 2(a) how many horizontal divisions will
now be required to display one full cycle of the 1000 Hz signal?

Number of divisions (calculated) = ____________

- Using the oscilloscope measure the number of required divisions and insert below. How does the result compare to the calculated number of divisions.

Number of divisions (measured) = ___________

- Change the horizontal sensitivity of the oscilloscope to I ms/div without touching any of the controls of the function generator. Using the results of Part 2(a), how many horizontal divisions will now be required to display one full cycle of the 1000 Hz signal?

Number of divisions (calculated) =   ___________

- Using the oscilloscope measure the number of required divisions and insert below. How does the result compare to the calculated number of divisions.

Number of divisions (measured) = _____________

- What was the effect on the appearance of the sinusoidal waveform as the horizontal sensitivity was changed from 0.2 ms/div. to 0.5 ms/div. and finally to 1 ms/div.

- Did the frequency of the signal on the screen change with each horizontal sensitivity? What conclusion can you draw from the results regarding the effect of the chosen horizontal sensitivity on
the signal output of the function generator?

- Given a sinusoidal waveform on the screen, review the procedure to determine its frequency. Develop a sequence of steps to calculate the frequency of a sinusoidal waveform appearing on the screen of an oscilloscope.

### Lab Task 1.3:

Vertical Sensitivity:

- Do not touch the controls of the function generator but set the sensitivity of the scope to 0.2 ms/div. and the vertical sensitivity to 2 V/div. Using this latter sensitivity, calculate the peak-to-peak value of the sinusoidal waveform on the screen by first counting the number of vertical divisions between peak values and multiplying by the vertical sensitivity.

Peak-to-peak value (calculated) = ____________

- Change the vertical sensitivity of the oscilloscope to 0.5 V/div. and repeat Part 2(a)

Peak-to-peak value (calculated) = ____________

- What was the effect on the appearance of the sinusoidal waveform as the vertical sensitivity was changed from 2 V/div. to 0.6 V/div.?

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Did the peak-to-peak voltage of the sinusoidal signal change with each vertical sensitivity? What conclusion can you draw from the results regarding the effect of changing the vertical sensitivity on the output signal of the function generator?

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Can the peak or peak-to-peak output voltage of a function generator be set without the aid of an auxiliary instrument such as an oscilloscope or DMM? Explain.

### Lab Task 1.4:

- Make all the necessary adjustments to clearly display a 6000-Hz 6Vp-p sinusoidal signal on the oscilloscope. Establish the zero volt line at confer of the screen. Record the chosen sensitivities:

Vertical sensitivity = 	______________

Horizontal sensitivity = _____________

- Draw the waveform on Fig. 1.1 carefully noting the required number of horizontal and vertical divisions. Add vertical and horizontal dimensions to the waveform using the chosen sensitivities listed above.

GRAPH:

Figure 1.1 Graph for Lab task 1.4, part (b)

- Calculate the period of the waveform on the screen using the number of horizontal divisions for a full cycle as shown.

T (calculated) =_______________

- Repeat Part 3(a) for a 200-Hz 0.8 Vp-p sinusoidal waveform on Fig. 1.2.

Vertical sensitivity = _____________

Horizontal sensitivity = ___________

T(calculated) = __________________

GRAPH:

Figure 1.2 Graph for Lab task 1.4, part (d)

- Repeat Fart 3(a) for a 100-kHz 4 Vp-p square wave on Fig. 1.3. Note that a square wave is called for

Vertical sensitivity = _____________

Horizontal sensitivity = ___________

T (calculated) = __________________

GRAPH:

Fig 1.3 Graph for Lab Task 1.4, Part (e)

## Lab Task-2:

## Pre-Lab: Familiarize yourself with diode

Introduction

Diode is a semiconductor device that, only allow current flow in one direction. The schematic diagram is shown in Figure 1.4, where the line denotes cathode or the N-material while the base is the anode or the P-material. Current flows from P to N or anode to cathode.

Stripe

Cathode

Anode

+

-

Figure 1.4 Diode Symbol

There are many specifications for each type of diode, the most important two are:

(1) PIV (Peak inverse Voltage) maximum voltages the diode can tolerate in reverse direction.

(2) IF (Forward Current) maximum forward current though diode when it is conducting.

Diodes have small impedance to current flow in one direction (forward-biased) and large impedance in the reverse-biased mode. When diodes fail they either short-circuit (pass current in both directions – i.e. low resistance in both directions) or open-circuit (do not pass current at all). Since the low impedance path is the one from anode to cathode, one needs to know which end is which.

Diodes are widely used in applications such as mixers, detectors, protection circuits. In this experiment you will investigate few applications of diodes such as AND gate, halfwave rectfier and Zener limiter. Diode limiters are wave shaping circuits in that they are used to prevent signal voltage from going above or below certain levels. Because of this clipping capability, the limiter is also called clipper.

Most modern· day digital multimeters can be used to determine the operating condition of a diode. They have a scale-denoted  by  a  diode  symbol  that  will indicate  the  condition  of a  diode  in the  forward  and  reverse-bias  regions.  If connected to establish a  ' forward bias condition  the  meter  will  display  the forward voltage across the  diode  at  a current- level  typically  in the neighbourhood  of 2  rnA.  If connected  to establish a  reverse-bias  condition  an "OL"  should appear on the  display  to  support  the  open-circuit  approximation frequently  applied  to  this  region. If the  meter does not  have the  diode-checking  capability  the  condition  of the diode can  also be checked by  obtaining  some  measure  of the  resistance  level  in  the  forward  and  reverse-bias region. Both techniques for checking diode will be introduced in the first part of the experiment. The current-volt (I-V) characteristics of a silicon or germanium diode have the general shape shown in Fig. 2.2.  Note the change in scale for both the vertical and horizontal axes.

In the reverse-biased  region the  reverse saturation currents  are  fairly  constant  from  0  V  to the  Zener  potential. In the forward-bias region the current increases quite rapidly with increasing diode voltage.  Note that  the curve  is rising  almost vertically  at a  forward-biased voltage  of less  than  1  V. 'The  forward-biased diode  current will  be limited solely  by the  network  in which  the  diode  is connected  or by the  maximum current or the power rating of the  diode.

Figure 1.5 Diode I-V Characteristics Curve

Tasks

- Review the INTRODUCTION section above.

- Simulate the diode characteristic using PSpice for comparison with experimentally measured results.

## In-Lab

Testing of diode with an analog Multimeter

To verify the diode is good or bad measure a DC forward resistance and DC reverse resistance. Good diode show low forward resistance and very high reverse resistance. Ratio of reverse and forward resistance should be 1000:1. If meter needle show deflection, the Red lead with terminal of diode show Cathode and vice versa.

Testing ordinary diode using a digital Multimeter

To check a silicon diode using a digital Multimeter, put the Multimeter selector switch in the diode check mode. Connect the positive lead of Multimeter to the anode and negative lead to cathode of the diode. If Multimeter displays a voltage between 0.6 to 0.7, we can assume that the diode is healthy. This is the test for checking the forward conduction mode of diode. The displayed value is actually the potential barrier of the silicon diode and its value ranges from 0.6 to 0.7 volts depending on the temperature.

Now connect the positive lead of Multimeter to the cathode and negative lead to the anode. If the Multimeter shows an infinite reading (over range), we can assume that the diode is healthy. This is the test for checking the reverse blocking mode of the diode.

Working of Diode in Forward & Reverse Biasing

### Lab Task 2.1:

Forward Bias

Figure 1.6

Procedure:

Assemble the circuit on proto board of diode, resister (1 kΩ) and variable power supply in series as given below.

  - Construct the network in Fig 1.6 with the supply (E) set at 0 V. Record the measure value of resistor.

  - Increase the supply voltage until VR reads 0.1 V. Then measure VD and insert its voltage in Table 1.1. Calculate the value of the corresponding current ID.

| VR | 0.1 | 0.2 | 0.3 | 0.4 | 0.5 | 0.6 | 0.7 | 0.8 | 0.9 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VD |  |  |  |  |  |  |  |  |  |  |  |  |
| ID= VR/ R |  |  |  |  |  |  |  |  |  |  |  |  |

Table 1.1

| VR | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VD |  |  |  |  |  |  |  |  |  |  |  |  |
| ID= VR/ R |  |  |  |  |  |  |  |  |  |  |  |  |

  - Sketch the waveform of the voltage across the diode and the current across the diode. This step will develop the characteristic curve of solid state conventional diode.

GRAPH:

Figure 1.7

### Lab Task 2.2:

Reverse Bias

Figure 1.8

Assemble the circuit on proto board of diode, resister and variable power supply in series as mention above.

        - In Fig 1.8 reverse bias condition has been established. Since the reverse saturation current will be relatively small, a large resistance of 1MΩ is required if the voltage across the resistance is to be of measureable amplitude. Record the measureable value of R.

        - Measure the voltage VR . Calculate the reverse saturation current from Is=VR(RM||R). The internal resistance of DMM (RM) is included because of the large amplitude of resistance R. A typical value of 10 MΩ is taken.

RM =

VR ( measured) =

Is ( calculated) =

Rubric for lab assessment

| The student performance for the assigned task during the lab session was: |  |  |  |
| --- | --- | --- | --- |
| Excellent | The student completed all the tasks and showed the results without any help of the instructor. | 4 |  |
| Good | The student completed all the tasks and showed the results with minimal help of the instructor. | 3 |  |
| Average | The student partially completed the task and showed results. | 2 |  |
| Worst | The student did not complete the task. | 1 |  |

Instructor Signature: __________________________         Date: __________________

# LAB #2: To construct a half wave and full wave rectifier circuit and display waveform using hardware tools

## Objectives

- To understand the behavior of a half wave rectifier circuit and display the output

- To understand the behavior of a full wave rectifier circuit and display the output

Lab Task-1:

Pre-Lab

Introduction

When AC signal is applied to a forward biased diode. The diode conducts for half positive or negative cycle and remains off for other half cycle. Diode converts the AC signal to Pulsating DC that can be observed on oscilloscope screen. The Primary function of half wave rectification is to establish a DC level from a sinusoidal input signal that has zero average (DC) level. DC voltage level in half wave rectification is equal to 31.8% of the peak voltage Vm considering the ideal diode.

Figure 2.1

Tasks

- Design a half wave rectification circuit using limiting resistor R, load resistor RL and diode. Graphs of Input voltage vi, output voltage vo, and current iR are shown in Fig. 2.1. Also find the values of limiting resistor R and load resistor RL.

Figure 2.2 Input voltage vi, output voltage vo and current iR

- Construct the circuit in Figure 2.3. Apply sinusoidal input voltage with 4V amplitudes and 1 kHz Frequency.

Figure 2.3

## In-Lab

### Lab Task 1.1:

- Design a half wave rectifier circuit that could completely clip off negative half of input ac signal. Consider 1000Hz 8 Vp-p sinusoidal input voltage.

- Plot the sinusoidal input on the graph of Fig 2.3. Determine the chosen vertical and horizontal sensitivities.

GRAPH:

Figure 2.4

Vertical Sensitivity =

Horizontal Sensitivity =

- Using the Oscilloscope with the AC-GND-DC coupling switch in the DC position, obtain the voltage Vo and sketch the waveform on Fig 2.3. Before viewing Vo be sure that to set the Vo = 0 V.

GRAPH:

Figure 2.5

### Lab task 1.2:

  - Design a half wave rectifier circuit that could completely clip off positive half cycle of the input ac signal.

Figure 2.6

  - Plot the input ac voltage signal in Fig. 2.7. Determine the chosen vertical and horizontal sensitivities.

GRAPH:

Figure 2.7

Vertical Sensitivity =  _____________

Horizontal Sensitivity =

  - Using the Oscilloscope with the AC-GND-DC coupling switch in the DC position, obtain the voltage Vo and sketch the waveform on Fig 2.8. Before viewing Vo be sure that to set the Vo = 0 V

GRAPH:

Figure 2.8

### Lab Task 1.3:

- Construct the network of Fig 2.9.Use R=2.2 kΩ

Figure 2.9

- Determine the theoretical output voltage for Fig 2.9 and sketch the waveform on Fig 2.10 for one cycle.  Indicate the maximum and minimum values on the output waveform.

GRAPH:

Fig 2.10

Vertical Sensitivity = _______________

Horizontal Sensitivity =

- Using the oscilloscope with the coupling switch in DC position obtain the voltage Vo and sketch the wave form on Fig 2.11 using the same sensitivities as in Part b.

GRAPH:

Fig 2.11

Vertical Sensitivity = _______________

Horizontal Sensitivity =

## Lab Task 2:

## Pre-Lab

Introduction

The objective of the lab is to reacquaint you with the fundamentals of AC (alternating current) and DC (direct current) voltages as well as introduce you to the basics of AC to DC conversion through the use of diode rectifiers.

The first section of the power supply in Figure 2.6, after the AC voltage source, is the transformer. It is responsible for converting the AC signal from a standard wall outlet down to a 12 VAC signal. Most DC power supplies maintain a voltage much less than 120 volts, so the transformer stage is necessary to get the AC source amplitude down to a more reasonable level.

When AC signal is applied to the rectifier circuit the diode D1and D4 are on for positive half cycle due to forward bias to produce output as replica of input at the same time, the diodes D2 and D3 remains open due to reverse bias.

When negative half cycle of input signal is applies to the rectifier circuit the diodes D1 and D4 is off due to reverse bias but diode D2 and D3 are on due to forward bias. Use capacitor across the load and see the effect of it. Use voltage regulator LM7805 here for output voltage regulation.

Figure 2.12

Design full wave bridge rectifier circuit that could generate an output voltage as shown in Fig. 2.13.

Figure 2.13

## In-Lab

Lab Task 2.1: Threshold Voltage

Choose one of the four silicon diodes you received and determine the threshold voltage, VT using the diode checking capability of DMM.

VT =

Figure 2.14

- Measure the rms voltage at the transformer secondary using DMM set to AC. Record that rms value below. Does it differ from the rated 12.6V

Vrms (measured) =

- Calculate the peak value of secondary voltage using the measured (Vpeak = 1.414 Vrms)

Vpeak (calculated) = __________

- Sketch the expected output waveform Vo on Fig 2.15. Choose a vertical and horizontal sensitivity based on the amplitude of the secondary voltage.

GRAPH:

Figure 2.15

Vertical Sensitivity =

Horizontal Sensitivity =

- Using the Oscilloscope with coupling switch in the DC position obtain the waveform for Vo and record on Fig 2.16. Use the same sensitivities employed in part c and be sure to preset Vo = 0 V.

GRAPH:

Figure 2.16

Vertical Sensitivity =

Horizontal Sensitivity =

- How do the waveform of part (c) and part (d) compare?

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Lab Task 2.2:     
   a. Determine the DC level of full-wave rectified waveform of Fig.2.15.

                                                                                                                VDC (calculated) =

- Measure the DC level of the output waveform using the DMM and calculate the present difference between the measured and calculated values.

                                                                                                      VDC (measured) =
                                                                                        
                                                                                                      ( % Difference ) =

Lab Task 2.3:

- Replace diode D3 and D4 in Fig 2.14 by 2.2kΩ resistors and forecast the appearance of the output voltage Vo .Sketch the waveform of Vo on Fig 2.17 and label the magnitude of minimum and maximum values.

GRAPH:

Figure 2.17

Vertical Sensitivity =

Horizontal Sensitivity

- Sketch the expected output waveform Vo using Oscilloscope in Fig 2.18. Choose a vertical and horizontal sensitivity based on the amplitude of secondary voltage.

GRAPH

Figure 2.18

Vertical Sensitivity =

Horizontal Sensitivity =

- How do the waveform of part (a) and part (b) compare?

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

### Lab Task 2.4:

- Determine the DC level of full-wave rectified waveform of Fig.2.17.


                                                                                                    VDC (calculated) =

- Measure the DC level of the output waveform using the DMM and calculate the present difference between the measured and calculated values.


 
                                                                                                    VDC (measured) =
                                                                                        
                                                                                                    ( % Difference )

Rubric for lab assessment

| The student performance for the assigned task during the lab session was: |  |  |  |
| --- | --- | --- | --- |
| Excellent | The student completed all the tasks and showed the results without any help of the instructor. | 4 |  |
| Good | The student completed all the tasks and showed the results with minimal help of the instructor. | 3 |  |
| Average | The student partially completed the task and showed results. | 2 |  |
| Worst | The student did not complete the task. | 1 |  |

Instructor Signature: __________________________         Date: __________________

# LAB # 3: To construct a zener diode based circuit and sketch its I-V characteristics

## Objectives

- To assemble a circuit where zener diode behaves as a regulator.

- To display and sketch the I-V graph for a zener diode

## Pre-Lab

Introduction

A zener diode is a special kind of diode which allows current to flow in the forward direction in the same manner as an ideal diode, but will also permit it to flow in the reverse direction when the voltage is above a certain value known as the breakdown voltage, "zener knee voltage" or "zener voltage." The device was named after Clarence Zener, who discovered this electrical property.

Zener diodes are heavily doped silicon diodes that, unlike normal diodes, exhibit an abrupt reverse break-down at relatively low voltages. The Zener diode is designed to operate in reverse breakdown region. Zener diode is used for voltage regulation purpose. Zener diodes are designed for specific reverse breakdown voltage called Zener breakdown voltage (Vz). The value of Vz depends on amount of doping Zener diodes are available in various families (according to their general characteristics , encapsulations and power ratings) with reverse breakdown (Zener) voltages in the range 2.4V to 200 V.

Figure 3.1

Tasks

- Design a voltage regulator that will maintain an output voltage of 20V across a 1 kΩ load with an input that will vary between 30V and 50V. That is, determine the proper value of Rs and the maximum current Izm.

- Simulate all the circuits in the lab on PSpice.

- Write down all the calculated values of all lab tasks before coming to the lab.

## In-Lab

Lab Task 1: To Plot I-V Characteristics

Figure 3.2

  - Construct the circuit of Fig 3.1 and set the DC supply to 0 V, use R=100 Ω and record the measured value of R.

  - Set the DC supply (E) to the value appearing in the table and measure both VZ and VR.

                                                                     Table 3.1

| E(V) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VZ (V) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VR (V) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IZ= VR/R (mA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - This step will develop the characteristic curve of Zener diode. Since the Zener region is in third quadrant to complete diode characteristic curve, place a minus sign in front of each level of IZ and VZ for each data point. With this convention in mind plot the data of the table 3.1 on the graph. Choose an appropriate scale for IZ and VZ as determined by the range of values for each parameter.

GRAPH:

Figure 3.3

Lab Task 2: Zener as a voltage regulator

  - Construct the network of Fig 3.4, use R= 1 kΩ and RL= 1 kΩ. Record the measure value of each resistor.

Figure 3.4

  - Determine whether the Zener diode is in “on” state that is operating in Zener breakdown region. For the diode in “on ” state calculate the expected value of VL, VR , IR , IZ , IL.

VL (calculated ) =

VR  (calculated) =

IR (calculated) =

IL (calculated) =

IZ (calculated) =

  - Energize the network of Fig 3.2 and measure the value of VL,VR , IR , IZ , IL.

VL ( measured ) =

VR  (measured) =

IR (measured) =

IL (measured) =

IZ (measured) =

  - Change RL to 1.2 kΩ and determine whether the Zener diode is in “on” state that is operating in Zener breakdown region. For the diode in “on ” state calculate the expected value of VL, VR , IR , IZ , IL.

VL (calculated ) =

VR  (calculated) =

IR (calculated) =

IL (calculated) =

IZ (calculated) =

  - Energize the network of Fig 3.4 and measure the value of VL,VR , IR , IZ , IL.

VL ( measured ) =

VR  (measured) =

IR (measured) =

IL (measured) =

IZ (measured) =

  - Change RL to 1.5 kΩ and determine whether the Zener diode is in “on” state that is operating in Zener breakdown region. For the diode in “on ” state calculate the expected value of VL, VR , IR , IZ , IL.

VL (calculated ) =

VR  (calculated) =

IR (calculated) =

IL (calculated) =

IZ (calculated) =

  - Energize the network of Fig 3.2 and measure the value of VL,VR , IR , IZ , IL.

VL ( measured ) =

VR  (measured) =

IR (measured) =

IL (measured) =

IZ (measured) =

  - Change RL to 2.2 kΩ and determine whether the Zener diode is in “on” state that is operating in Zener breakdown region. For the diode in “on ” state calculate the expected value of VL, VR , IR , IZ , IL.

VL (calculated ) =

VR  (calculated) =

IR (calculated) =

IL (calculated) =

IZ (calculated) =

  - Energize the network of Fig 3.4 and measure the value of VL,VR , IR , IZ , IL.

VL ( measured ) =

VR  (measured) =

IR (measured) =

IL (measured) =

IZ (measured) =

  - Change RL to 3.3 kΩ and determine whether the Zener diode is in “on” state that is operating in Zener breakdown region. For the diode in “on ” state calculate the expected value of VL, VR , IR , IZ , IL.

VL (calculated ) =

VR  (calculated) =

IR (calculated) =

IL (calculated) =

IZ (calculated) =

  - Energize the network of Fig 3.4 and measure the value of VL,VR , IR , IZ , IL.

VL ( measured ) =

VR  (measured) =

IR (measured) =

IL (measured) =

IZ (measured) =

  - Determine the minimum value of RL required to ensure that Zener is in “on” state.

RL ( calculated ) =

Rubric for lab assessment

| The student performance for the assigned task during the lab session was: |  |  |  |
| --- | --- | --- | --- |
| Excellent | The student completed all the tasks and showed the results without any help of the instructor. | 4 |  |
| Good | The student completed all the tasks and showed the results with minimal help of the instructor. | 3 |  |
| Average | The student partially completed the task and showed results. | 2 |  |
| Worst | The student did not complete the task. | 1 |  |

Instructor Signature: __________________________         Date: __________________

# LAB # 4: To display the output of diode based clipper circuit using hardware tools

## Objectives

- To measure the output dc and ac voltages of diode based series clipper circuit using digital storage oscilloscope (DSO).

- To measure the output dc and ac voltages of diode based shunt clipper circuit using digital storage oscilloscope (DSO).

## Lab Task-1:

## Pre-Lab

Introduction

It is frequently necessary to modify the shape of various waveforms for use in instrumentation, controls, computation, and communications. Wave shaping is often achieved by relatively simple combinations of diodes, resistors, and voltage sources. Such circuits are called clippers, limiters, amplitude selectors, or slicers. Clipper circuits are primarily used to prevent a waveform from exceeding a particular limit, either positive or negative. For example, one may need to limit a power supply’s output voltage so it does not exceed +5 V. The most widely used wave shaping circuit is the rectifier, which you have previously studied.

In electronics, a clipper is a circuit design to prevent the output of a circuit from exceeding a predetermined voltage level without distorting the remaining part of the applied waveform.

Diode clippers could be used at the inputs of small-signal instruments to protect against accidental application of large input signals. This is a low-current application, so you use signal diodes, not current diodes.

Figure 4.1 shows a positive clipper circuit. As indicated, the output voltage has the entire positive half-cycles clipped off. The circuit works as follows: During the positive half-cycle of the input voltage, the diode turns on. For an ideal diode, the output voltage is zero. For an actual diode the output voltage is equal to Vγ, the cut-in voltage of the diode.

During the negative half-cycle, the diode is reverse-biased and can be approximated by an open circuit. In many clippers, the load resistor, RL, is much larger than the series resistor, R. In which case, essentially all of the negative half-cycle voltage appears at the output through voltage-divider action. If RL and R are comparable, then on the negative half-cycle, the output voltage would be given by

Vo = Vp1 = Vp • (RL / (RL + R))

Since the first Vγ volts are used to begin conduction in the diode, the output signal is clipped near Vγ, rather than at 0V. If the diode polarity is reversed, the result is a negative clipper that removes the negative half cycle. In this case, the clipping levels occur near -Vγ.

Figure 4.1 Positive clipper circuit: (a) Sinusoidal input to clipper circuit; (b) A positive clipper       circuit; (c) Output of ideal positive clipper circuit; and (d) Output of actual positive clipper circuit

Tasks

- Design a clipping circuit with input voltage vi = 40 V p-p that can generate an output voltage vo as shown in figure 4.2.

Figure 4.2 output voltage vo  of required clipping circuit

- Simulate all the circuits in PSpice and perform the transient analysis before coming to the lab.

- Write down all the calculated values in the lab.

## In-Lab

Task 1.1: Determine Threshold voltage

Determine the threshold voltage for the silicon and germanium diodes using the diode-checking capability of the DMM or a curve tracer. Round off to hundredths place when recording in the designated space below. If the diode-checking capability or curve tracer is unavailable assume VT= 0.7 V for the silicon diode and 0.3 V for the germanium diode.

VT(Si) = __________________

VT(Ge) = __________________

Task 1.2: Parallel Clippers

- Construct the clipping network of figure 4.3 with R=2.2 kΩ. Record the measured resistance value of the cell. Note that the input is an 8 VP-P square wave at a frequency of 1000Hz.


                                                                  Figure 4.3

- Using the measured values of R, E, and VT calculate the voltage Vo when the applied square wave is +4V. What is the level of Vo? Show all the steps of your calculations to determine Vo.

Vo (calculated) = __________

- Repeat part 1.2(b) when the applied square wave is -4V.

- Using the results of parts 1.2(b) and 1.2(c) sketch the expected waveform for Vo   in Fig. 4.4

GRAPH:

Figure 4.4

- Using the sensitivities provided in part 1.2(d) set the input square wave and record Vo on Fig 4.5 using the oscilloscope. Be sure to preset the Vo = 0V line using the GND position of the coupling switch (and the DC position to view the waveform)

GRAPH:

Figure 4.5

How does the waveform of Fig 4.5 compare with the predicted results of Fig 4.4?

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Reverse the battery of Fig 4.3 and using the measured values of R, E and VT, calculate the level of Vo for the time interval when Vi= +4V

Vo(calculated) = __________

- Repeat part 1.2(f) for the time interval when Vi= -4V

VO(calculated) = __________

- Using the results of parts 1.2(f) and 1.2(g) sketch the expected waveform for Vo using the horizontal axis of Fig 4.6 as the Vo = 0V line. Use the same sensitivities provided in part 1.2(d).

GRAPH:

Figure 4.6

- Set the input square wave and record Vo on fig 4.7 using the oscilloscope. Be sure to preset the Vo = 0V line using the GND position of the coupling switch (and the DC position to view the waveform).

- How does the waveform of Fig 4.6 compare with the predicted results of Fig 4.7?

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

GRAPH:

Figure 4.7

Task 1.3: Series Clippers

- Construct the circuit of Fig 4.8 using R=1 MΩ. Record the measured resistance value and the DC level of the D cell. The applied signal is 8VP-P square wave at a frequency of 1000Hz.

Figure 4.8

- Using the measured values of R, E, and VT calculate the voltage Vo for the time interval when Vi = +4V.

VO(calculated) = __________

- Using the measured values of R, E, and VT calculate the voltage Vo for the time interval when  Vi = - 4V.

VO(calculated) = __________

- Using the results of parts 1.3(b) and 1.3(c) sketch the expected waveform Vo using the horizontal axis of Fig 4.9 as the  Vo = 0V line. Insert your chosen vertical and horizontal sensitivities below:

GRAPH:

Figure 4.9

Vertical sensitivity = __________________

Horizontal sensitivity = __________________

- Using the sensitivities chosen in part 1.3(d) set the input square wave and record Vo on Fig 4.10 using the oscilloscope. Be sure to preset the  Vo = 0V line using the GND position of the coupling switch (and the DC position to view the waveform)

GRAPH:

Figure 4.10

- How does the waveform of Fig 4.9 compare with the predicted results of Fig 4.10?

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Reverse the battery of Fig 4.8 and using the measured values of R, E, and VT calculate the level of Vo for the time interval when  Vi = +5V.

VO(calculated) = __________

- Repeat part 1.3(f) for the time interval when Vi= -4V

VO(calculated) = __________

- Using the results of part 1.3(f) and 1.3(g) sketch the expected waveform for Vo using the horizontal axis of Fig 4.11 as the Vo = 0V line. Use the following sensitivities:

Vertical: 2V/cm

Horizontal: 0.2ms/cm

GRAPH:

Figure 4.11

Rubric for lab assessment

| The student performance for the assigned task during the lab session was: |  |  |  |
| --- | --- | --- | --- |
| Excellent | The student completed all the tasks and showed the results without any help of the instructor. | 4 |  |
| Good | The student completed all the tasks and showed the results with minimal help of the instructor. | 3 |  |
| Average | The student partially completed the task and showed results. | 2 |  |
| Worst | The student did not complete the task. | 1 |  |

Instructor Signature: __________________________         Date: __________________

# LAB # 5: To display the output of diode based clamper circuit using hardware tools

## Objectives

- To measure the output voltages of diode based positive and negative clamper circuits using digital storage oscilloscope (DSO).

- To measure the output voltages of diode based biased clamper circuits using digital storage oscilloscope (DSO).

## Pre-Lab

Introduction

Clampers are designed to “clamp” an alternating input signal to a specific level without altering the peak to peak characteristics of the waveform. Clampers are easily distinguished form clippers in a way that they include a capacitive element. A typical clamper will include a capacitor, diode, and resistor with some also having a dc battery. The best approach to the analysis of the clampers is to use step by step approach. The first step should be an examination of the network for that part of the input signal that forward biased the diode. Choosing this part of the input signal will save time and some unnecessary confusion. With the diode forward biased the voltage across the capacitor and across the output terminals can be determined. For the rest of the analysis it is then assumed that the capacitor will hold on to the charge and voltage level established during this interval of the input signal. The next part of the input signal can then be analyzed to determine the effect of the stored voltage across the capacitor and the open-circuit state of the diode.

The analysis of a clamper can be quickly checked by simply noting whether the peak-to-peak voltage of the output signal is the same as the peak-to-peak voltage of the applied signal. This check is not sufficient to be sure the entire analysis was correct but it is a characteristic of clampers that must be satisfied.

In Figure 5.1 a positive dc clamper is shown. The clamper operates as follows: During the negative half-cycle of the input voltage, the diode turns on. At the negative peak, the capacitor charges up to Vp with the polarity shown and the output voltage is zero. As the voltage grows beyond the negative peak, the diode shuts off.

Tasks

- Design a clamping circuit which is able to perform the function as shown in Figure 5.1

Figure 5.1

- Plug-in all the circuits on trainer.

- Write down all the calculated values in the lab.

## In-Lab

### Lab Task 1.1:

Part 1: Determine the threshold voltage for the silicon diode using the diode checking capability of the DMM or a curve tracer. If either approach is unavailable assume VT = 0.7V.

Part 2: Clampers (R, C, Diode Combination)

- Construct the network of Fig 5.2 using R=100 kΩ and record the measured value of R.

Figure 5.2

- Using the value VT from lab task 1.1, part 1 calculate VC and VO for the interval of Vi that causes the diode to be in “on” state

VC(calculated) = __________

VO(calculated) = __________

- Using the results of lab task 1.1, part 2(b) calculate the level of VO after Vi switches to the other level and turns the diode “off”.

- Using the results of lab task 1.1, part 2 (b) and 2 (c) sketch the expected waveform for VO in Fig 5.3 for one cycle of Vi. Use the horizontal centre axis as the Vo = 0V line. Record the chosen vertical and horizontal sensitivities below:

GRAPH:

Figure 5.3

Vertical Sensitivity = ______________________

Horizontal Sensitivity = ______________________

- Using the sensitivities of lab task 1.1, part 2 (b) use the oscilloscope to view the output waveform VO. Be sure to preset the VO = 0V line on the screen using the GND position of the coupling switch (and the DC position to view the waveform). Record the resulting waveform on Fig 5.4.

GRAPH:

Figure 5.4

- How does the waveform of Fig 5.3 compare with the expected waveform of Fig 5.4?

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Reverse the diode of Fig 5.2, determine the levels of VC and VO for the interval of Vi that causes the diode to be in “on” state.

VC(calculated) = __________

VO(calculated) = __________

- Using the results of lab task 1.1, part 2(f) calculate the level of VO after Vi switches to the other level and turns the diode “off”.

VO(calculated) = __________

- Using the results of lab task 1.1, part 2(f) and 2(g) sketch the expected waveform for VO in Fig 5.5 for one cycle of Vi. Use the horizontal centre axis as the Vo = 0V line. Record the chosen vertical and horizontal sensitivities below:

GRAPH:

Figure 5.5

Vertical Sensitivity = ______________________

Horizontal Sensitivity = ______________________

- Using the sensitivities of part lab task 1.1, 2(h) use the oscilloscope to view the output waveform VO. Be sure to preset the VO = 0V line on the screen using the GND position of the coupling switch (and the DC position to view the waveform). Record the resulting waveform on Fig 5.6.

GRAPH:

Figure 5.6

- How does the expected waveform of Fig 5.5 compare with the waveform of Fig 5.6?

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Lab Task 1.2: Clampers (R, C, Diode Combination with a DC battery)

- Construct the network of Fig 5.7 using R=100 kΩ and record the measured value of R and Vo.

Figure 5.7

- Using the value VT from lab task 1.1, part 1 calculate VC and VO for the interval of Vi that causes the diode to be in “on” state

VC(calculated) = __________

VO(calculated) = __________

- Using the results of lab task 1.2(b) calculate the level of VO after Vi switches to the other level and turns the diode “off”.

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Using the results of lab task 1.2 (b) and lab task 1.2 (c) sketch the expected waveform for VO in Fig 5.8 for one cycle of Vi. Use the horizontal centre axis as the Vo = 0V line. Record the chosen vertical and horizontal sensitivities below:

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

GRAPH:

Figure 5.8

Vertical Sensitivity = ______________________

Horizontal Sensitivity = ______________________

- Using the sensitivities of lab task 1.2 (b) use the oscilloscope to view the output waveform VO. Be sure to preset the VO = 0V line on the screen using the GND position of the coupling switch (and the DC position to view the waveform). Record the resulting waveform on Fig 5.9.

GRAPH:

Figure 5.9

How does the waveform of Fig 5.8 compare with the expected waveform of Fig 5.9 ?

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Reverse the diode of Fig 5.7, determine the levels of VC and VO for the interval of Vi that causes the diode to be in “on” state.

VC(calculated) = __________

VO(calculated) = __________

- Using the results of lab task 1.2 (f) calculate the level of VO after Vi switches to the other level and turns the diode “off”.

VO(calculated) = __________

- Using the results of lab task 1.2 (f) and lab task 1.2 (g) sketch the expected waveform for VO in Fig 5.10 for one cycle of Vi. Use the horizontal centre axis as the Vo = 0V line. Record the chosen vertical and horizontal sensitivities below:

GRAPH:

Figure 5.10

Vertical Sensitivity = ______________________

Horizontal Sensitivity = ______________________

- Using the sensitivities of lab task 1.2 (h) use the oscilloscope to view the output waveform VO. Be sure to preset the VO = 0V line on the screen using the GND position of the coupling switch (and the DC position to view the waveform). Record the resulting waveform on Fig 5.11.

GRAPH:

Figure 5.11

- How does the waveform of Fig 5.10 compare with the expected waveform of Fig 5.11?

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Rubric for lab assessment

| The student performance for the assigned task during the lab session was: |  |  |  |
| --- | --- | --- | --- |
| Excellent | The student completed all the tasks and showed the results without any help of the instructor. | 4 |  |
| Good | The student completed all the tasks and showed the results with minimal help of the instructor. | 3 |  |
| Average | The student partially completed the task and showed results. | 2 |  |
| Worst | The student did not complete the task. | 1 |  |

Instructor Signature: __________________________         Date: __________________

# LAB # 6: To Sketch the input and output characteristics of common base BJT Transistor using hardware tools

## Objectives

Part 1: To sketch the input characteristics for a fixed output voltage of common base BJT Transistor using digital multi-meter (DMM).

Part 2:  To sketch the output characteristics for a fixed input current of common base BJT Transistor using digital multi-meter (DMM)

## Pre-Lab

Introduction

A Bipolar junction transistor (BJT) is a three terminal device capable of amplifying an ac signal (see Figure 1). The three terminals are called base (B), emitter (E), collector (C), and come in two flavours NPN (On a NPN transistor arrow is not pointed IN (as shown in Fig. 6.1) and PNP. The middle letter indicates the type of material used for the base, while outer letters indicate the emitter and collector material. The sandwiched materials produce two pn junctions. These two junctions form two diodes-the emitter-base diode and base-collector diode.

Figure 6.1 BJT pin configuration

BJTs are current amplifiers. A small base current is amplified to a larger current in the collector-emitter circuit. Consider first the NPN transistor shown in Fig. 6.1. If the base is at higher ( ≈0.6 volt) potential than the emitter then a current iB will flow into the base. The current into the collector is β times larger than the base current. The quantity β (usually called hFE in transistor data sheets) is a characteristic of the individual transistor and is typically in the range from 100 -500 for the types of transistors we will be using. The transistor can be thought of as a current amplifier device -- the current at the output (collector or emitter) is β times large than the current at the input (base). Another useful characteristic is the dc alpha.

For a transistor to amplify, power is required from dc sources. The dc voltages required for proper operation are referred to as bias voltages. The purpose of bias is to establish and maintain the required operating conditions despite variations between transistors or changes in the circuit parameters. For normal operation, the base-emitter junction is forward -biased and base-collector junction reverse-biased. Since the base emitter junction is forward-biased, it has characteristics of a forward-biased diode.

Task

- BJT IC – VCE characteristic curves

Simulate the circuit in PSpice as shown in Figure 6.1. Select Analysis →Setup → DC Sweep. Select VCE from 0 to 8V. The Sweep type is linear. Set Nested Sweep for IB from 0.1mA to 0.5mA. Mark X in the Enable Nested Sweep. Generate three curves for IB = 0.1mA, 0.3mA and 0.5mA.

Determine α and β from the curves for the following values of IB and VCE: IB = 0.1mA, 0.3mA and 0.5mA at VCE = 1V and VCE = 2.5V.

Figure 6.2

- Using the characteristics of Fig. 6.2, determine the resulting collector current if IE =4 mA and VCB =4V. Again find the collector current when IE =4 mA and VCB =16V. How have the changes in VCB affected the resulting IC? On an approximate basis, how are IE and IC related based on the results you have?

Figure 6.3 output or collector characteristics for a common base amplifier

BJT Biasing: Common Base Configuration

In common base, the base emitter is forward biased and the base collector junction is reverse biased. For input characteristics the emitter Current (IE) and base emitter Voltage (VBE) are variables and VCB is a parameter. Graphical relation between IE and VBE are similar of diode except VCB, will affect on IE and VBE. More the value of VCB more will be the emitter current (IE) because more the value of collector to base Voltage, more will be the minority carriers across the junction for fix value of base emitter Voltage. The emitter current varies with variation of base to emitter voltage (VBE) for fix value of collector to Base Voltage (VCB).

For output characteristics the emitter Current (IE) is taken as a parameter, base Collector voltage (VCB) and output Collector current are variables. For fix value of Emitter current the Collector current increases the ratio of IC/IE must also be increases by increase of VCB .The ratio of IC/IE is called α here α is not fix its values lies between 0-1 When VCB become negative the transistor is saturated because both junction are forwarded biased.

## In-Lab

Lab Task 1: Input Characteristics

Connect the circuit as shown in the circuit diagram of Fig 6.4. Measure the emitter current for different value of base to emitter voltages for fix value of VCB. Use Rc= 1 kΩ and RE  = 1 kΩ.

Figure 6.4: Common Base Configuration.

For VCB = 0V					         For VCB =5V

| S. No; VBE; IE. | S. No; VBE; IE. |
| --- | --- |

For VCB = 10V

| S. No | VBE | IE |
| --- | --- | --- |

Plot the graph between VBE (X-axis) and emitter current IE (Y-axis) in Fig. 6.4. The families of curves show three variables. Note that each curve resembles forward biased diode characteristics as expected but in this case the given VBE, IE increases with increasing VCB.

GRAPH:

Figure 6.5 Input characteristic curve

Lab Task 2: Output Characteristics

It is the curve between collector current IC and collector-base voltage VCB at constant emitter current IE. Measure the collector current for different value of collector to base voltages for fix value of IE.

For IE = 1mA					        For, IE =2mA

| S. No; VCB; IC. | S. No; VCB; IC. |
| --- | --- |

For IE = 3mA					         For IE =4mA

| S. No; VCB; IC. | S. No; VCB; IC. |
| --- | --- |

Plot the graph between VCB (X-axes) and collector current IC (Y-axes). The family of curves shows three variables.

GRAPH:

Figure 6.6 Output characteristic curve

Rubric for lab assessment

| The student performance for the assigned task during the lab session was: |  |  |  |
| --- | --- | --- | --- |
| Excellent | The student completed all the tasks and showed the results without any help of the instructor. | 4 |  |
| Good | The student completed all the tasks and showed the results with minimal help of the instructor. | 3 |  |
| Average | The student partially completed the task and showed results. | 2 |  |
| Worst | The student did not complete the task. | 1 |  |

Instructor Signature: __________________________         Date: __________________

# LAB # 7: To sketch the input and output characteristics of Common Emitter BJT Transistor using hardware tools

## Objectives

Part 1: To sketch the input characteristics for a fixed output voltage of Common Emitter BJT Transistor using digital multi-meter (DMM)

Part 2: To sketch the output characteristics for a fixed input current of Common Emitter BJT Transistor using digital multi-meter (DMM)

## Pre-Lab

Introduction

In common emitter the base emitter is forward biased and the base Collector junction should be reverse biased. For input characteristics the base current (IB) and base emitter voltage (VBE) are variables and VCE is a parameter. More the value of VCE less will be the Emitter current (EE) because more is the value of Collector to Emitter voltage less will be the minority carriers cross the junction for fix value of collector emitter voltage. The base current varies with variation of base to emitter voltage (VBE) for fix value of collector to emitter voltage (VCE).

In common Emitter the Base Emitter is forward biased and the base collector junction should be reverse biased. For output characteristic curves the collector current (IC) and collector emitter voltage (VCE) are variables and IB is a parameter. The Collector current (IC) is in milli-ampere range and VCE are in volt range. The parameter IB is in µA range. The collector current varies with variation of Base to Emitter voltage (VBE) for fix value of base current (IB). In Common Emitter case the relation between IC and VCE are not similar as in common base output Characteristics. The curves of IB are not horizontal as those obtained for IE in common base configuration indicating that the Collector to Emitter voltage will influence the magnitude of the Collector current.

Tasks

- Simulate the circuit in Figure 7.1 in PSpice.

- You should be familiar with all the formulas related to common emitter configuration.

- Using the characteristics of Fig. 7.1

  - Find the value of IC corresponding to VBE =720 mV and VCE =+5V

  - Find the value of VCE and VBE corresponding to IC =3 mA and IB = 30 µA

Figure 6.1 Characteristics of silicon transistor in the common emitter configuration

## In-Lab

Lab Task 1: Connect the circuit as shown in the circuit diagram in Fig 7.2. Measure the base current for different value of base to emitter voltages for fix value of VCE.

Figure 7.2

For VCE = 1V				             	For VCE =10V

| S. No; VBE; IB. | S. No; VBE; IB. |
| --- | --- |

For VCE = 20V

| S. No | VBE | IB |
| --- | --- | --- |

Plot the graph between VBE (X-axis) and Emitter current IB (Y-axis). The family of curves shows three variables. Note that each curve resembles forward biased diode characteristics as expected but in this case the given VBE, IB decreases with increasing VCE.

GRAPH:

Figure 7.3

Lab Task 2: Output Characteristics

Measure the collector current for different value of collector to emitter voltages for fix value of IB.

For IB = 0µA					For, IB =10µA

| S. No; VCE; IC. | S. No; VCE; IC. |
| --- | --- |

For IB = 20µA

| S. No | VCE | IC |
| --- | --- | --- |

Plot the graph between VCE (X-axis) and Emitter current IC (Y-axis). The family of curves shows three variables.

GRAPH:

Figure 7.4

Rubric for lab assessment

| The student performance for the assigned task during the lab session was: |  |  |  |
| --- | --- | --- | --- |
| Excellent | The student completed all the tasks and showed the results without any help of the instructor. | 4 |  |
| Good | The student completed all the tasks and showed the results with minimal help of the instructor. | 3 |  |
| Average | The student partially completed the task and showed results. | 2 |  |
| Worst | The student did not complete the task. | 1 |  |

Instructor Signature: __________________________         Date: __________________

# LAB # 8: To measure the quiescent operating point of Fixed and Emitter Biased BJTs using hardware tools

## Objectives

Part 1:  To measure the quiescent operating point of Fixed Biased BJTs using digital multi-meter (DMM)

Part 2: To measure the quiescent operating point of Emitter Biased BJTs using digital multi-meter (DMM)

## Pre-Lab

Introduction to Fixed and Emitter Biasing of BJTs

Bipolar transistors operate in three modes: cutoff, saturation, and linear. In each of these modes, the physical characteristics of the transistor and the external circuit connected to it uniquely specify the operating point of- the transistor. In the cutoff mode, there is only a small amount of reverse current from emitter to collector, making the device akin to an open switch. In the saturation mode, there is a maximum current flow from collector to emitter. The amount of that current is limited primarily by the external network connected to the transistor; its operation is analogous to that of a closed switch. Both of these operating modes are used digital circuits. For amplification with a minimum of distortion, the linear region of the transistor characteristics is employed. A DC voltage is applied to the transistor, forward-biasing the base-emitter junction and reverse-biasing the base-collector junction, typically establishing a quiescent point near or at the
center of the linear region. In the first part of this experiment, we will investigate the fixed-
bias network.

Tasks

- Simulate the circuit in Figure 8.1 and 8.2 in PSpice.

- You should be familiar with all the formulas related to fixed and emitter biasing of BJTs.

- Write down all the calculated values in this experiment before coming to the lab.

- Design a fixed bias circuit with VCC =14 V, VC =6 V, IB=40 µA and β=80. Determine

- IC

- RC

- RB

- VCE

## In-Lab

Lab Task 1.1: Design a fixed bias circuit using the 2N3904 transistor and implement it.

Figure 8.1

- Measure the voltage VBE and VRC

VBE (measured)  =  _______________

VRC (measured)  =  _______________

- Using the measured resistor values calculate the resulting base current using the equation:

IB = VRB/ RB   =   (VCC - VBE)/ RB

and the collector current using the equation

IC = VRC/ RC

The voltage VRB was not measured directly for determining IB because of the loading     effects of the meter across the high resistance RB

Insert the resulting values of IB. and Ic in Table 8.1.

- Using the results of step 1.1(c), calculate the value of β and record in Table 8.1. This value of beta will be used for the 2N3904 transistor throughout this experiment.

### Lab Task 1.2:

- Using the β determined in Part 1.1, calculate the currents IB and Ic for the network of Fig. 8.1 using the measured resistor values, the supply voltage, and the above measured value for VBE, That is, determine the theoretical values of IB and IC using the network parameters and the value of beta.

IB(calculated) = ____________

IC(calculated) = ____________

- How do the calculated levels of IB and IC compare to those determined from measured voltage levels in part 1.1(c)?

- Using the results of step 1.2(a) calculate the levels of VB, VC, VE, and VCE.

VB(calculated) = _____________

VC(calculated) = _____________

VE(calculated) = _____________

VCE(calculated) = _____________

- Energize the network of Fig 8.1 and measure VB, VC, VE, and VCE.

VB(measured) = _____________

VC(measured) = _____________

VE(measured) = _____________

VCE(measured) = _____________

- How do the measured values compare to the calculated levels of step 1.2(b)?

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Record the measured values of VCE in table 8.1

- The next part of the experiment will essentially be a repeat of a number of the steps above for a transistor with a higher beta. Our goal is to show the effects of different beta levels on the resulting levels of the important quantities of the network. First the beta level for the other transistor, specifically a 2N4401 transistor, must be determined. Simply remove the 2N3904 transistor from Fig. 8.1 and insert the 2N4401 transistor, leaving all the resistors and voltage VCC as to Part 1. Then-measure the voltages VBE and VRC and, using the same equations with measured resistor values, calculate the levels of IB and IC.

VBE(measured) = _____________

VRC(measured) = _____________

IB(measured) = _____________

IC(measured) = _____________

β (calculated) = ______________

- Record the levels of IB, IC, and beta in Table 8.1. In addition measure the voltage VCE and insert in Table 8.1.

Table 8.1

| Transistor type | VCE | IC | IB | Β |
| --- | --- | --- | --- | --- |
| 2N3904 |  |  |  |  |
| 2N4401 |  |  |  |  |

- Using this following equations calculate the magnitude (ignore the sign) of the percent change in each quantity due to a change in transistors. Ideally, the important voltage and current levels should not change with a change in transistors. The fixed-bins configuration, however, has a high sensitivity to changes in' beta as will be reflected by the results. Place the results of your calculations in Table 8.2.

%ΔVCE = [VCE (4401) – VCE(3904)]/ VCE (3904)  x 100%

%ΔIC = [IC (4401) – IC (3904)]/ IC (3904)  x 100%

% ΔIB = [IB (4401) – IB (3904)]/ IB (3904)  x 100%

%Δβ = [β(4401) – β(3904)]/ β(3904)  x 100%

Table 8.2

Percentage changes in β,IC. VCE,and IB

| %Δβ | % ΔIB | %ΔIC | %ΔVCE |
| --- | --- | --- | --- |

Part 2: Lab Tasks

In this part, the emitter bias circuit is investigated.

The emitter bias configuration in Fig. 8.2 can be constructed using a single or a dual power supply. Both configurations offer increased stability over the fixed bias of previous Experiment. In particular, if the beta times of the transistor times the resistance of the emitter resistor is large compared to the resistance of the base resistor, the emitter current becomes essentially independent of the beta of the transistor. Thus, if we exchange transistors in a properly designed emitter-bias circuit, the changes inIc andVCE should be small.

In the first part of this experiment, we will investigate the fixed-bias network.

Lab Task 2.1: Determining Beta

- Construct the network of Fig. 8.2 using  2N39Q4 transistor, Insert the measured resistor values:

Figure 8.2

- Measure the voltages VB andVc .

,	.	VB(measured)______________________

Vc  (measured)____________________

- Using the results of Part (b) and the measured resistor values calculate the resulting base currents IB and Ic using the following equations:

IB=Vcc-VB/RB

And IC=VRC/RC

Record in table 8.2

IB(measured)=_____________________

IC(measured)=_____________________

- Using the results of step 2.1(c) calculate the value of  β and record in Table 8.2. This value of beta will be used for the 2N3904 transistor throughout the experiment.

β=_____________________

### Lab Task 2.2:

- Using the β determined in Part 2.1, calculate the values of IB and IC for the network of Fig. 8.2 using measured resistor values and the supply voltage VCC. In other words, perform a theoretical analysis of the network. Insert the results in Table 8.3.

IB(measured)=----------------------

IC(measured)=----------------------

- How do the calculated values compare with the measured values of part 2 of lab Task 1.1(c)?

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Using the β determined in Lab task 2.1 calculate the levels of VB, VC, VE, VBE and VCE and insert in Table 8.1.

Table 8.3 (Calculated Values)

| Transistor type | VCE | VB | VC | VE | VBE | VCE | IB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2N3904 |  |  |  |  |  |  |  |
| 2N4401 |  |  |  |  |  |  |  |

Table 8.4 (Measured values)

| Transistor type | VCE | VB | VC | VE | VBE | VCE | IB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2N3904 |  |  |  |  |  |  |  |
| 2N4401 |  |  |  |  |  |  |  |

- Calculated from the measured values

2N3904 β=_____________________

2N4401 β=_____________________

- Replace the 2N3904 transistor of Fig. 8.2 with the 2N4401 transistor and measure the resulting voltages VB andVRC, Then calculate the currents IB and IC using measured resistance values. Finally calculate the value of β for this transistor. This will be the
value of beta used for the 2N4401 transistor throughout this experiment. Record the levels of IB, IC and β in Table 8.3.

VB(measured)=______________

VRC(measured)=______________

- Using the beta determined in step 1(c), perform a theoretical analysis of Fig. 8.2 with the 2N4401 transistor. That is, calculate the levels of IB, IC, VB, VC, VE and VCE insert in Table 8.4.

Rubric for lab assessment

| The student performance for the assigned task during the lab session was: |  |  |  |
| --- | --- | --- | --- |
| Excellent | The student completed all the tasks and showed the results without any help of the instructor. | 4 |  |
| Good | The student completed all the tasks and showed the results with minimal help of the instructor. | 3 |  |
| Average | The student partially completed the task and showed results. | 2 |  |
| Worst | The student did not complete the task. | 1 |  |

Instructor Signature: __________________________         Date: __________________

# LAB # 9:  To construct a voltage-divider biased common-emitter circuit and measure its Q point voltage and current using digital multimeter

## Objectives

To measure the quiescent operating conditions of the voltage divider bias BJT configurations

## Pre-Lab

Introduction

In the previous bias configurations the bias current ICQ and voltage VCEQ were a function of the current gain (β) of the transistor. However, since β is temperature sensitive, especially for silicon transistors, and the actual value of beta is usually not well defined, it would be desirable to develop a bias circuit that is less dependent, or in fact, independent of the transistor beta. The voltage-divider bias configuration of Fig. 11.2 is such a network. If analyzed on an exact basis the sensitivity to changes in beta is quite small. If the circuit parameters are properly chosen, the resulting levels of ICQ and VCEQ can be almost totally independent of beta. The level of IBQ will change with the change in beta, but the operating point on the characteristics defined by ICQ and VCEQ can remain fixed if the proper circuit parameters are employed.

Tasks

- Simulate the circuit in Fig. 9.2 in PSpice.

- Write down all the calculated values in the experiment before coming to the lab.

- Design a voltage divider bias circuit using a supply of 24 V, a transistor with a beta of 110, and an operating point of ICQ=4 mA and VCEQ=8 V. Choose VE=18VCC. Use standard values.

## In-Lab

Lab Task 1:

Determining β: Construct the network of Fig. 9.1 using the 2N3904 transistor. Insert the measured resistance values.

Figure 9.1

- Measure the voltage VBE and VRC

VBE (measured)  =  _______________

VRC (measured)  =  _______________

- Using the measured resistor values calculate the resulting base current using the equation:

IB = VRB/ RB   =   (VCC - VBE)/ RB

and the collector current using the equation

IC = VRC/ RC

The voltage VRB was not measured directly for determining IB because of the loading     effects of the meter across the high resistance RB

Insert the resulting values of IB. and Ic in Table 9.1.

- Using the results of lab task 1(b) , calculate the value of β and record in Table 9.1, This value of beta will be used for the 2N3904 transistor throughout this experiment.

Lab Task 2: Voltage-Divider Configuration	.

- Construct the network of Fig. 9.2 using the 2N3904 transistor. Insert the measured value of each resistor.

Figure 9.2

R1 (measured) = ____________

R2 (measured) = ____________

RC (measured) = ____________

RE (measured) = ____________

- Using the beta determined in Part 1 for the 2N3904 transistor, calculate the theoretical levels of VB, VE, VC,IE, IC, and IB, for the network of Fig. 9.2. Insert the results in Table 9.1.

Table 9.1

| 2N3904 | VB | VE | VC | VCE | IC | IB | IE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Calculated |  |  |  |  |  |  |  |
| Measured |  |  |  |  |  |  |  |

- Energize the network of Fig. 9.2 and measure VB, VE, VC and VCE. Record their values in Table 9.1. In addition, measure the voltages VR1 and VR2. Try to measure the quantities to the hundredth or thousandth place, Calculate the currents IE and IC and the currents I1 and I2 (using I1 = VR1, /R1 and I2 = VR2, /R2) from the voltage readings and measured resistor values. Using the results for I1 and I2, calculate the current IB using Kirchhoff’s current law. Insert the calculated current levels for IE, IC, and IB in Table 9.1.

How do the calculated and measured values of Table 9.1 compare? Are there any significant differences that need to be explained?

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Insert the measured value of VCE and calculated values of Ic and IB from step 2(c) in Table 9.2 along with the magnitude of beta from Part 1.

- Replace the 2N3904 transistor of Fig 9.2 with the 2N4401 transistor. Then measure the voltages VCE, VRC, VR1, and VR2.  Again, be sure to read VR1 and VR2 to the hundredth or thousandth place to ensure an accurate determination of IB. Then calculate IC, I1,I2, and determine IB. Complete table 9.2 with the levels of VCE, IC,IB, and beta for this transistor

Table 9.2

| Transistor type | VCE | IC | IB | β |
| --- | --- | --- | --- | --- |
| 2N3904 |  |  |  |  |
| 2N4401 |  |  |  |  |

- Calculate the percent change in β, IC, VCE and IB from the data of table 9.2. Use the formulas appearing in step 2(e), Eq. 9.1, and record your results in table 9.3

Percentage changes in β, IC, VCE and IB

Table 9.3

| Transistor type | VCE | IC | IB | β |
| --- | --- | --- | --- | --- |
| 2N3904 |  |  |  |  |
| 2N4401 |  |  |  |  |

Rubric for lab assessment

| The student performance for the assigned task during the lab session was: |  |  |  |
| --- | --- | --- | --- |
| Excellent | The student completed all the tasks and showed the results without any help of the instructor. | 4 |  |
| Good | The student completed all the tasks and showed the results with minimal help of the instructor. | 3 |  |
| Average | The student partially completed the task and showed results. | 2 |  |
| Worst | The student did not complete the task. | 1 |  |

Instructor Signature: __________________________         Date: __________________

# LAB # 10: To Design a Voltage-Divider Biased Common-Emitter Amplifier with Fully Bypassed Emitter Resistance and Analyze the effect of Load on its AC Operation Using Hardware and Software Tools

## Objectives

- To design voltage-divider biased common emitter amplifier for given specified conditions using standard design criteria

- To construct the designated circuit and compare the measured voltage gain with calculated one using mathematical equations, digital oscilloscope, function generator and circuit simulator

- To analyze the effect of load on ac operation of small-signal common emitter amplifiers digital oscilloscope, function generator and circuit simulator

## Pre-Lab

Introduction

The design of an amplifier needs a transistor, a power source, some resistors, and some capacitors. There are multiple configurations using NPN transistors, but here, we design the "common emitter amplifier" because it allows us to have high voltage gain.
The design process begins with appropriate biasing of the circuit. This would enable us to predict the output swing that we required. The schematic diagram of voltage divider common emitter amplifier with emitter resistor R_E fully bypassed is shown below:

Figure 10.1: Voltage Divider Bias Common Emitter Amplifier

Design Procedure

To work properly in an amplifier circuit a bipolar junction transistor (BJT) must be properly biased and operate in the active mode. This is achieved by a proper resistor network. The following conditions should be established:

- The emitter is biased at about 10% of V_CC voltage to achieve temperature stability. This gives the value of RE (assume that emitter and collector quiescent currents are the same). For this select:

VE=VCC10

For common emitter configuration, IQ=IC  and  IC=IE , the value of RE should be                                                              
RE=VEIE

- The collector is biased half way between ground and VCC so that the output can swing both way equally. This gives the value of RC for a given collector quiescent current (with no input signal).

VQ=VCE=VCC2

RC=VCC-VQIC

- For the base voltage VB,

VB=R2R1+ R2VCC=VE+0.7V

- Find the ratio of R1:R2 to put the base ~0.7V above the emitter. For this select R1 and R2 as large as possible but still sensitive to the DC condition βRE≥10 R2 so the system is not loaded down. The current following through R2 should be 10% of IC. i.e.

I2=IC10

Using this find the value of R2 and then R1 using the following formulas:

R2= VBI2

R1=VCC-VBI2

- Find value of achievable voltage gain is given as

Av=-RC∥rCβ		re=26mVIE

Effect of Load on AC Operation

If we attach a load resistance RL to the amplifier of Figure 1.1, the maximum output voltage is reduced in this case. This happens because the effect of output resistance rout has to be considered and the open-ended output voltage vo is divided into rout and RL. Here (VQ, IQ) is the Q-point and. This introduces ac load-line to the ac operation of the amplifier defined by the following intercepts on the two axis.

To avoid clipping and ensure maximum equal swings of both half-cycles, the following condition should be satisfied:

It can be observed that  results in clipping of positive half-cycle while  results in negative half-cycle clipping.

## In-Lab Tasks

Lab Task 1: Design of voltage divider biased CE Amplifier with Emitter resister fully bypassed

Design Specification

Design specifications for common emitter amplifier are:

| Sr. No. | Quantity | Calculated Value |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |

Method

- Calculate values of all biasing resistors using the procedure stated in pre-lab and design specification provided to you.

- Use the values of coupling and bypass capacitors directed by your instructor and draw resulting circuit diagram.

- Implement the circuit on bread board for DC analysis (without attaching the source and capacitors) and verify that it is properly biased.

- After getting satisfactory biasing, connect 1 kHz AC voltage source and capacitor to the amplifier as shown in schematic diagram in pre-lab section.

- Now adjust the voltage source to get maximum unclipped output voltage vo. Also note value of input voltage vi for this undistorted output voltage vo.

- Calculate the obtained voltage gain using measured values and compare it with calculated one

Circuit Diagram

Measurement and Calculation

Table 10.1: Bias Design

| Sr. No. | Component | Calculated Value | Standard Value and Tolerance |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |

Table  10.2

| Sr. No. | Quantity | Measured Value | Calculated Value |
| --- | --- | --- | --- |
| 1 | VB |  |  |
| 2 | VE |  |  |
| 3 | VC |  |  |
| 4 | IC=IE |  |  |
| 6 | vo (distortion-free maximum peak) |  |  |
| 7 | vS (distortion-free maximum peak for 2) |  |  |
| 8 | Av |  |  |

Lab Task 2: Effect of load on its AC operation

Method

- Implement the circuit shown in Figure 10.1 on the breadboard using your designed values.

- Now choose different values of RL and calculate corresponding ac load-line intercepts for the Q-point which provided maximum unclipped peak-to-peak output swing.

- Measure peak-to-peak load voltage vL for each value of RL in Step 2 and observe any clipping if occurs. Categorize the clipping as positive clipping, negative clipping, double clipping and no clipping.

- Record your observations in Table 10.3.

- Calculate the value of RL from (3) that gives maximum unclipped peak-to-peak swing of the load voltage and verify for the given circuit on breadboard.

- Measure maximum unclipped symmetric peak-to-peak load voltage swing and record in Table 10.4.

Measurement and Calculation

Table 10.3: Effect of load on AC operation

| RL (kΩ) | AC Loadline Description |  | (V) | Clipping |
| --- | --- | --- | --- | --- |
|  | (V) | (mA) |  |  |

Table 10.4: Maximum unclipped load voltage

| Maximum unclipped peak-to-peak load voltage, vL (p-p) (Calculated) | V |
| --- | --- |
| Maximum unclipped peak-to-peak load voltage, vL (p-p) (Measured) | V |

Lab Task 3: Software Simulation

- Use a circuit simulator for analysis of your designed common emitter circuit and discuss difference between practical and simulation results if any

- Design the voltage divider common emitter amplifier with emitter resistor RE fully bypassed using exact approach for same operating conditions mentioned in experiment.

What sort of limitations would different AC loads impose on the performance of the circuits?

Rubric for lab assessment

| The student performance for the assigned task during the lab session was: |  |  |  |
| --- | --- | --- | --- |
| Excellent | The student completed all the tasks and showed the results without any help of the instructor. | 4 |  |
| Good | The student completed all the tasks and showed the results with minimal help of the instructor. | 3 |  |
| Average | The student partially completed the task and showed results. | 2 |  |
| Worst | The student did not complete the task. | 1 |  |

Instructor Signature: __________________________         Date: __________________

# LAB # 11: To Construct the Class A Power Amplifier and Compute its Efficiency Using Hardware and Software tools

## Objectives

- To construct the series fed class A power amplifier and measure DC and AC voltages using oscilloscope, function generator and circuit simulator

- To compute the efficiency of series fed class A power amplifier using digital oscilloscope, function generator and circuit simulator

- To construct the class A power amplifier with capacitor –coupled load and measure DC and AC voltages using digital oscilloscope, function generator and circuit simulator

- To compute the efficiency of the class A power amplifier with capacitor –coupled load using digital oscilloscope, function generator and circuit simulator

## Pre-Lab

Introduction

Power Amplifiers

The amplifiers which amplify voltage as well as current are called power amplifiers. A typical example is a common-emitter amplifier with any biasing configuration. Since the power amplifiers are aimed to deliver a large amount of power to the load, they must be able to withstand ambient effects to avoid melting, burning or damaging. For this purpose, power transistors with a high power rating are used. They contain a large surface area and a metal case. In addition to this, care resistors power rating must be taken into account.

Power amplifiers operate under large-signal conditions. By large-signal condition we mean that the amplifier may operate in nonlinear region that is IC = βIB is no longer valid.

The performance of power amplifiers is measured in terms of its efficiency, η given by:

(1)

It is notable that power gain and efficiency are two separate performance measurement. As apparent from (1), the efficiency is measured with respect to the power drawn from dc source, but not ac source.

Since the power transistors themselves dissipate considerable power, it is also important to know, how much of the power is dissipated in the collector. Therefore, the performance of the power amplifier is also expressed in form of collector efficiency given by:

(2)

| Figure 11.1: Series-fed Class A Amplifier | Figure 11.2:Capacitor-coupled Class A Power Amplifier |
| --- | --- |

Power amplifiers are classified as Class A, B, C, D, E. Class A, Class B and Class C comprise of BJTs while

Class D and E use FETs. We will limit our lab work to Class A and Class B power amplifiers only. They are classified on the basis of output waveform as compared to sinusoidal input waveform.

Class A Power Amplifiers

An amplifier is said to be a Class A amplifier if the output remains in the active region (no saturation, no cut- off) for a full sinusoidal input cycle. Class A power amplifiers are used in three different configuration: series- fed, capacitor-coupled and transformer-coupled. We will limit ourselves to the first two configurations in the lab.

Efficiency of Series-fed Class A Power Amplifiers

In a series-fed configuration, the collector resistance RC is treated as the load resistance RL (RC = RL) as shown in Figure 11.1. For maximum efficiency, the transistors is operated at the midpoint of dc loadline so that an undistorted peak-to-peak swing can be achieved at the output i.e. the collector terminal. The efficiencies of this configuration is given as:

(3)

Where

= peak load current					= Q-point collector current

Imposing the condition of maximum undistorted peak-to-peak swing, i.e. VQ = VCC/2 and IL = IQ = VCC/2RC, the  maximum efficiency turns out to be 0.25.

The collector efficiency can be expressed as

(4)

Where. Its maximum value will therefore be 0.5 using the condition stated above.

Efficiency of Capacitor-coupled Class A Power Amplifiers

In a capacitor-coupled configuration, a separate load RL is connected to the collector terminal through a coupling capacitor as shown in Figure 11.2. It is obvious that now the peak collector current IP = IQ will be divided into RC RC and RL using current-divider rule as below:

(5)

The efficiency will be expressed similar to that of series-fed by substituting (5) into (3) as

(6)

To find out the maximum efficiency, we need a two-fold optimization: taking the Q-point to be optimum for load-attached case, and taking the value of RC that is followed by maximum power transfer theorem. We know that the optimum Q-point in this case (i.e. maximum undistorted peak-to-peak load voltage swing, VQ=IQrL) is expressed as:

(7)

Putting (7) into (6), we are left with

(8)

This is the efficiency if transistor is operated on its optimum Q-point. Imposing the second condition, it has been found that the efficiency will be maximum if the collector resistance is RL=RC/2. Substituting this value, the maximum efficiency turns out to be 0.0833.

### In-Lab Tasks

Lab Task 1: To measure the efficiency of a series-fed class A Power Amplifier

Method

- Implement the circuit of Figure 11.1 on breadboard using a power transistor but without connecting the source.

- Measure the Q-point collector voltage VQ.

- Calculate Q-point collector current IQ and verify it through measurement.

- Calculate the power drawn from dc source using.

- Connect a sinusoidal source of 10 kHz

- Adjust the source to measure largest undistorted peak voltage drop across the collector resistance RL and calculate the power delivered to it using.

- Calculate efficiency using (3). Note if it is maximum.

- Calculate collector efficiency using (4). Note if it is maximum.

- Record your observations and calculations in Table 11.1.

- Now, adjust dc supply in such a way that VQ = 0.5VCC. By this, we will try to achieve the maximum efficiency.

- Repeat steps iv-viii and record your observations and calculates in Table 11.2.

Note: Do not perform steps x and xi if step vii gives maximum efficiency.

Measurement and Calculation

Table 11.1: Efficiency of Series-Fed Class A Power Amplifier

| Sr. No. | Quantity | Measured Value | Calculated Value |
| --- | --- | --- | --- |
| 1 | VQ |  | Not required |
| 2 | IQ |  |  |
| 3 | VL |  | Not required |
| 4 | ɳ | Not required |  |
| 5 | Ηc | Not required |  |

Table 11.2: Maximum Efficiency of Series-Fed Class A Power Amplifier

| Sr. No. | Quantity | Measured Value | Calculated Value |
| --- | --- | --- | --- |
| 1 | VQ |  | Not required |
| 2 | IQ |  |  |
| 3 | VL |  | Not required |
| 4 | Ηmax | Not required |  |
| 5 | ηc,max | Not required |  |

Lab Task 2: To measure the efficiency of a Capacitors-coupled class A Power Amplifier

Method

- Implement the circuit in Figure 11.2 on breadboard but without connecting the source and the load.

- Repeat steps (ii) – (v) of Part I.

- Adjust the source to measure largest undistorted peak voltage drop across the load resistance RL and calculate the power delivered to it using.

- Repeat steps (vii) and (viii) of Part I

- Record your observations and calculations in Table 11.3.

- Calculate the optimum collector current using (7) or VQ=IQRL (as both represent the same condition).

- Remove the dc source and set the Q-point according to step (vi) through measurement.

- Repeat steps (iv) and (v) of Part I.

- Repeat steps (iii) and (iv).

- Record your observations and calculations in Table 11.4.

- Maintaining the Q-point set in step vii, select the values of RC and RL such that maximum power transfer condition of RL=RC/2 is satisfied. Do it by changing RL only.

- Calculate the efficiency using (8) and check if it is maximum.

- Record your observations and calculations in Table 11.5.

Note: Do not perform steps (vi)-(x) if step (iv) gives maximum efficiency. Proceed step (xi) onwards in such case.

Measurements and Calculations

Table 11.3: Efficiency of Capacitor-Coupled Class A Power Amplifier

| Sr. No. | Quantity | Measured Value | Calculated Value |
| --- | --- | --- | --- |
| 1 | VQ |  | Not required |
| 2 | IQ |  |  |
| 3 | VL |  | Not required |
| 4 | ɳ | Not required |  |
| 5 | Ηc | Not required |  |

Table 11.4: Efficiency of Capacitor-Coupled Class A Power Amplifier For Optimum Point

| Sr. No. | Quantity | Measured Value | Calculated Value |
| --- | --- | --- | --- |
| 1 | VQ=IQRL | Not Required |  |
| 2 | IQ |  |  |
| 3 | VL |  | Not required |
| 4 | Ηmax | Not required |  |

Table 11.5: Maximum Efficiency of Capacitor-Coupled Class A Power Amplifier For Maximum Power Transfer Case

| Sr. No. | Quantity | Calculated Value | Standard Value Used |
| --- | --- | --- | --- |
| 1 | RL=RC/2 |  | Not required |
| 2 | IQ |  |  |
| 3 | VL |  | Not required |
| 4 | Ηmax | Not required |  |

Lab Task 3: Software Simulation

- Implement the circuit of Figure 11.1 in a circuit simulator and compare your results with practical results.

Justify the deviation from theoretical maximum efficiency if it arises

Rubric for lab assessment

| The student performance for the assigned task during the lab session was: |  |  |  |
| --- | --- | --- | --- |
| Excellent | The student completed all the tasks and showed the results without any help of the instructor. | 4 |  |
| Good | The student completed all the tasks and showed the results with minimal help of the instructor. | 3 |  |
| Average | The student partially completed the task and showed results. | 2 |  |
| Worst | The student did not complete the task. | 1 |  |

Instructor Signature: __________________________         Date: __________________

# LAB # 12: To Design and Construct the Basic Linear Op-Amp Circuit using Hardware and Software Tools

## Objectives

- To design and select standard passive components of an op-amp for desired gain and sketches output waveform using theoretical concepts, digital oscilloscope, function generator and circuit simulator

- To design and select standard passive components of an op-amp for voltage summation and subtraction and sketches output waveform using theoretical concepts, digital oscilloscope, function generator and circuit simulator

## Pre-Lab

Linear Op-Amp

Introduction

Operational amplifier is one fundamental building block of analog circuits having very high gain. When used properly in negative feedback configurations, the overall closed‐loop transfer characteristic can be precisely set by stable passive components such as resistors, capacitors, and diodes, regardless of the potential variation of open‐loop parameters. Since there is a linear relation between input and output, we usually refer to these application circuits as linear applications. Negative feedback produces bounded input-bounded output stability.

A finite input voltage cannot produce an infinite output voltage. In our lab, we commonly use LM741 op amp for basic configurations. When used in a circuit with resistors and a necessary dependent source, the op amp can be used to sum, scale, subtract, and perform other useful functions. When used with inductors and capacitors, it is used in integrating and differentiating circuits.

7

Vn

2

-

6

Vp +

3

4

+Vcc

Vo

-Vcc

Figure 12.1: Circuit Symbol and pin configuration for an Op Amp

Figure 13.1 shows the schematic symbol and pin configuration of an op amp. A is the voltage gain. The inverting input is 𝑣𝑛, and the non-inverting input is 𝑣𝑝. The differential input is

𝑣𝑖𝑛  = 𝐴(𝑣𝑛 − 𝑣𝑝)

Where  , 𝑣𝑝 and 𝑣𝑜𝑢𝑡 are node voltages. This means they are always measured with respect to ground. The differential input 𝑣𝑖𝑛 is the difference of two node voltages, 𝑣𝑛 and𝑣𝑝.

Basic Op-Amp configuration

As an inverting amplifier the resistors are connected to the inverting input as shown in Figure 13.2 with output voltage.

vo= -R2R1v1

Figure 12.2: Inverting Amplifier

A non-inverting amplifier is shown in Figure 13.3 with output voltage given by

vo=1+R2R1v1

Figure 12.3: Non-Inverting Amplifier

Compensating Resistor RC

Transistor within op-amp must be biased so that has current values of base and collector current and collector to emitter’s voltages. The ideal op-amp has no input bias current, but in fact, practical op-amp has small input bias current typically in nano ampere range.

The most common compensation involves adding a resistor 𝑅𝑐 to the standard amplifier to cancel out bias currents [output offset]. Inverting and Non-inverting amplifier with a compensating resistor is given below:

| - Inverting Amplifier with RC | - non-inverting amplifier with RC |
| --- | --- |

Figure 12.4: Op-amp with RC

The compensation resistor 𝑅𝑐 causes a current, on the positive terminal, equal and opposite to current flowing into the negative terminal. So, any DC output offset caused by the inverting input is cancelled by the non- inverting input. The value of R3 should be equal to the parallel combination of 𝑅1 and𝑅2.

For inverting amplifier, compensating resistor should be

Rc=R2ǁrs+R1

Where rs appears series withR1.

For non-inverting amplifier, where source is connected directly to the + input, the sum of rs  and Rc should be approximately equal toR1ǁR2, which means

Rc=R2ǁR1-rs

A voltage follower can be obtained by connecting the output back to the inverting input as in Figure 13.5. It provides a gain of exactly unity.

vo= v1

Figure 12.5: Voltage Follower

Summation and Subtraction Circuits

To obtain output of the form

vo= a1v1 +a2v2+a3v3

We use op-amp as a summing amplifier. A summing amplifier (Voltage summation) can be made by more than one input connected through separate resistors as shown in Figure 13.6. It can be made for both inverting and non-inverting configuration. For inverting, the expression will be same and for non-inverting, negative sign will be placed with output expression.

The summing is performed at the negative input terminal in current domain due to the virtual ground property (note that the positive input is grounded). The weight for each parallel input is solely determined by the resistor connecting this input to the summing node. The output voltage is a linear combination of all the input voltages.

vo= -RfR1v1 +RfR2v2+RfR3v3

Figure 12.6: Summing Amplifier (Voltage Summation)

Where

Rc=RfǁR1ǁR2ǁR3

The voltage subtraction produce an output voltage that equals the mathematical difference between two inputs signals as shown in Figure 13.7.

The gains for the positive and negative inputs can be set differently by choosing different R1 and R2 values.

vo=1+R4R3R2R1+R2v1-R4R3v2

Figure 12.7: Voltage Subtractor

## In-Lab Tasks

Lab Task 1:  Inverting and Non-inverting Op-Amp

- Design an inverting op-amp with closed loop gain of 10 when R2=100kΩ

- Now take approximate values of calculated values of components obtained from step 1 and implement the inverting configuration by applying an input signal of 1Volt at f=10 kHz. Note down the values in table.

- Now connect a compensating resistor Rc with the non-inverting terminal of op-amp and note the readings.

- Repeat the step (i), (ii) and (iii) for non-inverting configuration for a gain of 11.

Measurement and Calculation

Table 12.1: Design values

| Components | Calculated | Standard Values |
| --- | --- | --- |
| R2 (Ω) |  |  |
| R1 (Ω) |  |  |
| Rc (Ω) |  |  |

Table 12.2: Inverting and Non-inverting Amplifier

| Values | Inverting |  |  |  | Non-inverting |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculated |  | Measured |  | Calculated |  | Measured |  |
|  | Without Rc | With Rc | Without Rc | With Rc | Without Rc | With Rc | Without Rc | With Rc |
| vi |  |  |  |  |  |  |  |  |
| vo |  |  |  |  |  |  |  |  |
| Gain |  |  |  |  |  |  |  |  |

Sketch Waveform

Sketch the waveform for vi and v0 for inverting and non inverting amplifier.

| - Inverting | - Non-inverting |
| --- | --- |

Lab Task 2:  Voltage Follower

- Design a voltage follower configuration and calculate the gain.

- Now take approximate values of calculated values of components obtained from step (i) and implement the configuration by applying an input signal of 1 Volt at f=10 kHz.

Measurement and Calculation

Table 12.3: Voltage Follower

| Parameters | Calculated | Measured |
| --- | --- | --- |
| vi |  |  |
| v0 |  |  |
| Gain |  |  |

Lab Task 3:  Voltage Summation

Method

- Design an op-amp that will produce an output equal to

vo=-4v1+v2+2v3

When	 v1=sinωt V	v2=+5 V	v3=-5 V

Select Rf=100kΩ 	f=10 kHz 	and        Rc as a compensated resistor.

- Now take approximate values of calculated values of components obtained from step (i) and implement the configuration by applying input signals as indicated in design requirement at f=10 kHz.

Measurement and Calculation

Table 12.4: Design values for Voltage Adder

| Components | Calculated | Standard Values |
| --- | --- | --- |
| Rf (Ω) |  |  |
| R1 (Ω) |  |  |
| R2 (Ω) |  |  |
| R3 (Ω) |  |  |
| Rc (Ω) |  |  |

Table 12.5:  Voltage Adder

| Parameters | Calculated | Measured |
| --- | --- | --- |
| v1 |  |  |
| v2 |  |  |
| v3 |  |  |
| v0 |  |  |

Sketch Waveform

Sketch output waveform for voltage summation as displayed on oscilloscope during lab.

Horizontal Sensitivity =______

Vertical Sensitivity =  ______

Lab Task 4:  Voltage Subtractor

Method

- Design an op-amp that will produce an output equal to	vo=0.5v1-0.4v2

When 		v1=2sinωt V		v2=+5 V

Select	R4=100kΩ	R2=20kΩ	and 	f=10 kHz

- Now take approximate calculated values of components obtained from step (i) and implement the configuration by applying input signals as indicated in design requirement at f=10 kHz

Measurement and Calculation

Table 12.6: Design Values for Voltage Subtractor

| Components | Calculated | Standard Values |
| --- | --- | --- |
| R4 (Ω) |  |  |
| R1 (Ω) |  |  |
| R2 (Ω) |  |  |
| R3 (Ω) |  |  |

Table 12.7:  Voltage Subtractor

| Parameters | Calculated | Measured |
| --- | --- | --- |
| v1 |  |  |
| v2 |  |  |
| vo |  |  |

Sketch Waveform

Sketch output waveform for voltage subtractor as displayed on oscilloscope during lab.

Horizontal Sensitivity =______

Vertical Sensitivity =  ______

Lab Task 3: Software Simulation

- Implement all above four circuits in any circuit simulator and compare yours results with practical results

- Attach simulated output waveform of all above mentioned circuits

Rubric for lab assessment

| The student performance for the assigned task during the lab session was: |  |  |  |
| --- | --- | --- | --- |
| Excellent | The student completed all the tasks and showed the results without any help of the instructor. | 4 |  |
| Good | The student completed all the tasks and showed the results with minimal help of the instructor. | 3 |  |
| Average | The student partially completed the task and showed results. | 2 |  |
| Worst | The student did not complete the task. | 1 |  |

Instructor Signature: __________________________         Date: __________________